# EV Simulation Decisions

Focused handoff: battery sizing and travel behavior

Scope. This outline covers battery sizing, commute travel, non-commute travel, and trip timing.

Out of scope. EV adoption and EV count will be set by the parametric study and are not decided here.

Small-sample caution. Keep the five-home simulation traceable at the individual-home level. Use sampled trip and vehicle characteristics rather than assuming the block average will represent a larger fleet.

# 1. Current model context

The five-home example currently uses fixed battery EV (BEV) inputs per home: battery capacity, maximum charger power, arrival/departure time, and arrival SOC. The next implementation step is to replace the fixed travel and SOC assumptions with documented battery and trip inputs. EV count remains a parametric-study input; however, the sizes of the EV’s still needs to be considered. I am thinking a default safe assumption would be sedan EV or SUV EV and ignore trucks. We could make this a function of the occupancy in the home too though. Every decision in this process should have a reference when possible otherwise, a well thought out reasoning to defend in review.

# 2. Battery capacity and sizing

Use vehicle class to select a battery-capacity distribution. PD’s starting ranges are 50–90 kWh for sedans and 120–170 kWh for trucks; the sampling rule and source still need to be selected.

Define the vehicle classes used by the simulation and map each class to a capacity distribution.

Choose uniform, triangular, empirical, or model-level sampling within each range.

Clarify gross versus usable capacity and the SOC limits passed to OCHRE.

Set vehicle-class energy use in kWh/mile, including any temperature or speed adjustment.

Define initial SOC, minimum SOC, maximum SOC, and how insufficient trip energy is handled.

Use NHTS vehicle fields only for travel/vehicle context; source battery capacity from vehicle specifications or another vehicle database.

Minimum battery inputs. Vehicle class; usable capacity (kWh); energy use (kWh/mile); initial/minimum/maximum SOC; and the reference for the source or rule for each value used.

# 3. Commute travel

Use county-level data to calibrate commute duration, then use NHTS to shape travel behavior and timing. NHTS is not designed for county-level estimates; its official FAQ identifies national, census-region, and census-division use as the supported geographic levels. When implemented, there should be a distribution of uncertainty for commute travel each day separated by weekday and weekend.

Sample commute duration by worker/household from the selected county distribution.

PD’s method: Choose a speed assumption and calculate one-way distance, or use observed trip miles where appropriate.

Decide whether work travel occurs five days per week, varies by weekday, or includes work-from-home days.

Convert distance to trip energy using the vehicle-class efficiency assumption.

Track the vehicle’s away-from-home interval so it cannot charge during the commute.

NHTS fields to review. PERSON: WORKER, WRKTIME, TIMETOWK, and work-from-home fields. TRIP: TRPMILES, TRVLCMIN, STRTTIME, TRAVDAY, and the trip/vehicle-use fields. Confirm exact 2022 names and codes in the data dictionary.

# 4. Non-commute travel

Use the NHTS TRIP file to estimate non-commute trip frequency, purpose, distance, duration, and departure time. Grocery travel can be represented using the shopping/buy-goods purpose category, then translated into a feasible trip only when the simulated vehicle is home. When implemented, there should be a distribution of uncertainty for non-commute travel each day separated by weekday and weekend.

Select the non-commute purposes to represent; begin with grocery/shopping and document whether it is always included or sampled.

Use trip purpose, trip miles, trip duration, start time, travel day, and vehicle-used indicators to build the input distributions.

Use NHTS weights when calculating population-level distributions, and filter negative special-code values before computing distances or durations.

Do not schedule a non-commute trip if the vehicle is already away or charging; sort trips into a daily sequence and enforce a home/away state.

Track non-commute miles and energy separately from the charging event for auditability.

NHTS fields to review. TRIP: TRIPPURP, WHYFROM, WHYTO, TRPMILES, TRVLCMIN, STRTTIME, TRAVDAY, TRPHHVEH, and occupancy fields such as TRPHHACC/TRPACCMP where available.

# 5. Trip timing

Create distributions around work departure/arrival and non-commute departure/return times. Preserve day-to-day variation, weekday/weekend differences, and the vehicle’s at-home eligibility.

Work timing: sample home-to-work departure and arrival using trip start time, trip duration, work arrival time, and commute duration.

Non-commute timing: sample departure and return by purpose; grocery trips should not all occur at one fixed time.

Use TRAVDAY/weekend fields to distinguish weekday and weekend behavior.

Use work-from-home fields to avoid creating a commute on a work-from-home day.

For a five-home block, decide whether schedules are independent or share common shifts caused by weekday, weather, or household routines.

# 6. Official NHTS input sources

Use 2022 NHTS V2.1 as the primary source unless the project has a reason to stay with 2017. The Downloads page contains the core files; the Documentation page contains the 2022 codebook, data dictionary, user guide, and derived-variable documentation.

| Simulation use | NHTS file / fields to extract | Official source |
| --- | --- | --- |
| Core data and weights | HOUSEHOLD, PERSON, VEHICLE, and TRIP files. Use the file-specific final weights; confirm names in the 2022 data dictionary. | NHTS Downloads; NHTS Documentation |
| Battery/vehicle context | VEHICLE fields such as vehicle type, make, model, and year. Use these to classify travel/vehicle context, not as a battery-capacity source. | NHTS Documentation; 2022 Data Dictionary |
| Commute travel | PERSON work fields plus TRIP miles, duration, start time, travel day, and vehicle-use fields. | 2022 User’s Guide; NHTS FAQ |
| Non-commute travel | TRIP purpose fields (TRIPPURP, WHYFROM, WHYTO), miles, duration, start time, travel day, vehicle use, and occupancy. | 2022 Derived Variables; 2022 User’s Guide |
| Trip timing | TRIP start/end or duration fields, TRAVDAY/weekend fields, and PERSON work-arrival/work-from-home fields. | NHTS Codebook / Data Dictionary; 2022 User’s Guide |

NHTS Downloads — 2022 V2.1 core data in CSV, SAS, and SPSS formats.

NHTS Documentation — 2022 codebook, data dictionary, user guide, and derived variables.

2022 NHTS User’s Guide — File structure, trip concepts, variables, weights, and special codes.

2022 Derived Variables — Definitions and derivation logic for trip-purpose and related fields.

NHTS FAQ — Geographic validity, weighting, trip purpose, travel day, and negative-value handling.

NHTS Codebook Browser — Variable lookup; use the downloadable 2022 codebook/data dictionary for 2022 fields.

Recommended workflow. Download the 2022 CSV files, confirm field names/codes in the data dictionary, filter invalid special codes, apply the appropriate file weight, and document the exact extraction query used for each distribution.